Multi-level Mapping: Real-Time Dense Monocular SLAM

(Green, Kyel, Peter, Nicholas)

Cameras are powerful sensors for robotic navigation because they provide rich environment information (colour, shape, texture, etc.) at high resolution and long ranges, while being lightweight, low-power, and inexpensive. Exploiting such sensor data for navigation tasks typically falls into the realm of monocular SLAM, where both the robot’s pose and a map of the environment are estimated concurrently from the imagery produced by a single camera mounted on the robot.

The basic approach to solve this problem is in accordance with two major observations, which apparently misguide us regarding the depth estimation of the low texture image:

- In the absence of high-frequency information such as sharp image gradients, depth estimation is typically unreliable

- In man-made environments, low-texture regions are often correlated with simple or planar surfaces.

Motivated by these observations, this method uses variable-resolution quadtrees to represent keyframe images, where we estimate the depth of each leaf in the quadtree, rather than each pixel in the image. The quadtree representation of the keyframe enables modelling of different image regions at a scale appropriate for the available texture: high-texture regions will be represented at the finest scale, while low-texture regions will be represented at coarser scales.

Multi-level mapping is sliced into 6 sections:

- Basic Dense Monocular SLAM: Given a sequence of images from a moving camera and the current designated keyframe, our goal is to estimate online, the current pose, the inverse depthmap, and the optimal pose.

- Depth estimation using quadtree keyframes (node, pixel): estimate and update using the magnitude of the image gradient, which is skipped in case the value falls below a certain predefined threshold. Thereafter, determination of the inverse depth (basically the reverse process).

- Hole filling: Holes come into existence when the aforementioned values of depth (nodes) estimation fail to occur. This is carried out by considering the spatial neighbours.

- Triangulation and Rasterization: Despite Depth-map updates on the variable resolution representation to enable spatial regularization and hole filling, it is essential to track the next incoming image and display point clouds using the best full-resolution keyframe depth-map, this taking the current variable resolution keyframe depth-map and recover the native value.

- Pose-graph optimization: we use the robust image and depth alignment method for generating constraints in the pose-graph in order to estimate the optimal keyframe poses and point cloud.

Hence, this method extends the semi-dense solution of to produce dense point clouds while being able to run online at 30 Hz. Depth estimation component intelligently distributes resources to image regions with hightexture while estimating low-texture regions at coarser resolutions, achieving a substantial speedup. In addition, this allows for low-frequency information to be exploited in the depth estimation. The noisy, multi-level inverse depthmaps are then smoothed using a variational regularizer(hole filling) before being triangulated and rasterized back up to the native image scale and added to a keyframe pose-graph. The increased keyframe density also improves tracking performance when combined with a global illumination term to account for lighting differences between frames.

PROBABILISTIC SLAM:

- In probabilistic form, the Simultaneous Localisation and Map Building (SLAM) problem requires that the probability distribution

- P (xk, m | Z0:k, U0:k, x0)

- The SLAM algorithm is now implemented in a standard two-step recursive (sequential) prediction (time-update) correction (measurement-update) form:

P (xk, m | Z0:k−1, U0:k, x0)= P(xk | xk-1, uk ) × P(xk-1, m| Z0:k-1, U0:k-1, x0)dxk-1

P (xk, m | Z0:k, U0:k, x0)= P (zk | xk , m) P(xk , m | Z0:k-1, U0:k , x0)P (zk | Z0:k-1, U0:k )

Should be computed for all times k.

- This probability distribution describes the joint posterior density of the landmark locations and vehicle state (at time k) given the recorded observations and control inputs up to and including time k together with the initial state of the vehicle.

Extended Kalmann Filter(EKF) SLAM:

- The basis for the EKF-SLAM method is to describe the vehicle motion in the form:

P(xk | xk−1, uk ) ⇐⇒ xk = f(xk−1, uk ) + wk

- where f (·) models vehicle kinematics and where wk are additive, zero mean uncorrelated Gaussian motion disturbances with covariance Qk .

- The observation model is de- scribed in the form

P(zk | xk, m) ⇐⇒ z(k) = h(xk, m) + vk

- where h(·) describes the geometry of the observation and where v  are additive, zero mean uncorrelated Gaussian observation errors with covariance Rk .

- There is generally no need to perform a time update for stationary landmarks

Observation Update

Where ∇h is the Jacobian of h evaluated at xk|k-1 and mk-1.

Odometry is the use of data from motion sensors to estimate change in position over time. It is used in robotics by some legged or wheeled robots to estimate their position relative to a starting location.

Demerits of EKF SLAM:

- Convergence of the map.

- Substantial computational efforts which grows with increasing landmarks.

- Data association problem is compounded in environments where landmarks are not simple points and indeed look different from different view-points

- Non-Linearity can be a significant problem in EKF-SLAM and leads to inevitable, and sometimes dramatic, inconsistency in solutions.

FastSLAM:

- The FastSLAM algorithm marked a fundamental conceptual shift in the design of recursive probabilistic SLAM.

- Previous efforts focused on improving the performance of EKF-SLAM, while retaining its essential linear Gaussian assumptions.

- FastSLAM with its basis on recursive Monte Carlo sampling, or particle filtering, was the first to directly represent the non-linear process model and non-Gaussian pose distribution.

- The high dimensional state-space of the SLAM problem makes direct application of particle filters computationally infeasible. However, it is possible to reduce the sample-space by applying Rao-Blackwellisation (R-B), whereby a joint state is partitioned according to the product rule:

P(x1, x2) = P(x2|x1)P(x1)

and, if P(x2|x1) can be represented analytically, only P(x1) need be sampled x1(i)~ P(x1).

- The joint distribution, therefore, is represented by the set:

EKF-SLAM: Linearises non Linear Problem hencecomplexity is quadratic

FastSlam: Non Linear solution hence complexity is linear

- The joint SLAM state may be factored into a vehicle component and a conditional map component:

- Here the probability distribution is on the trajectory X0:k rather than the single pose xk because, when conditioned on the trajectory, the map landmarks become independent.

- This is a key property of FastSLAM, and the reason for its speed; the map is represented as a set of independent Gaussians, with linear complexity, rather than a joint map covariance with quadratic complexity.

- The general form of a R-B particle filter for SLAM is as follows. We assume that, at time k − 1, the joint state is represented by:

- For each particle, compute a proposal distribution, conditioned on the specific particle history, and draw a sample from it.

  - This new sample is (implicitly) joined to the particle history

- Weight samples according to the importance function

  - The numerator terms of this equation are the observation model and the motion model, respectively.

If necessary, perform resampling.

3-D SLAM:

- Implementing SLAM in 3-D is, in principle, a straightforward extension of the 2-D case. However, it involves significant added complexity due to the more general vehicle motion model and, most importantly, greatly increased sensing and feature modelling complexity

- There exist three essential forms of 3-D SLAM.

- 2-D SLAM with additional map building capabilities in the third dimension.

  - For example, horizontal laser-based SLAM with a second orthogonal laser mapping vertical slices.

  - This approach is appropriate when the vehicle motion is confined to a plane.

- 2-D SLAM to 3-D, with the extraction of discrete landmarks and joint estimation of the map and vehicle pose.

  - This has been implemented with monocular vision sensing and permits full six degree-of-freedom motion.

- The third form involves an entirely different SLAM formulation, where the joint state is composed of a history of past vehicle poses.

  - At each pose, the vehicle obtains a 3-D scan of the environment and the pose estimates are aligned by correlating the scans.

Dynamic Environments:

- Real world environments are not static. They contain moving objects, such as people, and temporary structures that appear static for a while but are later moved, such as chairs and parked cars.

- In dynamic environments, a SLAM algorithm must somehow manage moving objects;

  - It can detect and ignore them;

  - It can track them as moving landmarks

But it must not add a moving object to the map and assume it is stationary.

- The conventional SLAM solution is highly redundant. Landmarks can be removed from the map without loss of  consistency, and it is often possible to remove large numbers of landmarks with little change in convergence rate

- Simultaneous estimation of moving and stationary landmarks is very costly due to the added predictive model. For this reason, the solution implemented in first involves a stationary SLAM update followed by separate tracking of moving targets.

- Error Margins: The robot should not have an error of more than 2cm per meter moved and 2 degrees per 45 degrees turned

Sensors used:

RGB-D Sensors:

Involves using infrared based camera.

- One such camera is the Kinect, which integrates depth and colour data resulting in up to 300,000 colour points/frame of an image or a set of images hence it is a preferred choice compared to other RGB-D sensors which cost more and are less precise.