Specs of the robot

- Weight of the robot:

- Kinect camera: 200grams

- Motor: 125 grams

- Powerbank: 400 grams

- Chassis and some other stuffs: 100 grams

So the total weight of the robot will be around 800gms to 1Kg.This weight of the robot is distributed over the wheels of the robot.

- Torque rating of the motor:

The root should overcome its weight to move in a direction. So  the torque required for the wheels motor can be calculated by basic kinematic torque equation that Torque= Force*Radius

T= k*N*r

Where k: coefficient of static friction=0.6(for a plane surface)

N: average weight acting over the wheel(here the weight acting on caster wheels is neglected as center of mass is located close to the wheels)=0.45kg

r: radius of the wheel=0.025m

Substituting the values in the equation we get torque required for the motor to be around 0.3Kg-cm.

- Height of the robot:

The height of the motor will be sum of the diameter of the wheel and the vertical length of the Kinect camera.

So, H=5cm+1.5cm= 6.5cm

- Speed of the motor:

The linear velocity of the wheels is related to its angular velocity by v=r*w

Where r is the radius of the wheel and w is the angular velocity.

Taking the linear velocity of the robot to be 1m/s the required angular velocity is calculated to be around 400 rpm.

Considering the losses due to friction in contact we will need a 1000 rpm dc motor.

- Power requirements:

The raspberry pi 3 will be needing 2amps, 11000mah,5v supply. It can be powered be a required powerbank. The kinect camera is supplied by an auxiliary supply.